CPUlator ARM Assembly Program
Trace and Analysis

Submission Task

# Program

global _start

_start:
    mov r1, #18       @ r1 = ?
    mov r2, #6        @ r2 = ?
    add r3, r1, r2    @ r3 = ?
    sub r4, r1, r2    @ r4 = ?
    lsl r5, r2, #1    @ r5 = ?
    mul r6, r3, r2    @ r6 = ?
    add r7, r4, r5    @ r7 = ?
    b .               @ halt

# Question 1 — Trace Table

The following table shows the register changed after each of the seven required instructions, along with its hexadecimal and decimal value.

| Step | Instruction | Register Changed | Hex Value | Decimal Value |
| --- | --- | --- | --- | --- |
| 1 | mov r1, #18 | r1 | 0x00000012 | 18 |
| 2 | mov r2, #6 | r2 | 0x00000006 | 6 |
| 3 | add r3, r1, r2 | r3 | 0x00000018 | 24 |
| 4 | sub r4, r1, r2 | r4 | 0x0000000C | 12 |
| 5 | lsl r5, r2, #1 | r5 | 0x0000000C | 12 |
| 6 | mul r6, r3, r2 | r6 | 0x00000090 | 144 |
| 7 | add r7, r4, r5 | r7 | 0x00000018 | 24 |

## Step-by-Step Calculations

Step 1

r1 = 18
18 = 0x12

Step 2

r2 = 6
6 = 0x06

Step 3

r3 = r1 + r2
   = 18 + 6
   = 24
   = 0x18

Step 4

r4 = r1 - r2
   = 18 - 6
   = 12
   = 0x0C

Step 5

r5 = r2 << 1
   = 6 << 1
   = 12
   = 0x0C

A left shift by 1 is equivalent to multiplying by 2: 6 × 2 = 12.

Step 6

r6 = r3 × r2
   = 24 × 6
   = 144
   = 0x90

Step 7

r7 = r4 + r5
   = 12 + 12
   = 24
   = 0x18

## Final Register State

| Register | Hex Value | Decimal Value |
| --- | --- | --- |
| r1 | 0x00000012 | 18 |
| r2 | 0x00000006 | 6 |
| r3 | 0x00000018 | 24 |
| r4 | 0x0000000C | 12 |
| r5 | 0x0000000C | 12 |
| r6 | 0x00000090 | 144 |
| r7 | 0x00000018 | 24 |

## CPUlator Final Register Screenshot

Insert the CPUlator Registers panel screenshot here, showing the final register state after all seven required instructions have executed.

[ INSERT CPUlator SCREENSHOT HERE ]

# Question 2 — Explain Step 6

The instruction in Step 6 is:

mul r6, r3, r2

At this point, r3 contains 24. The value of r3 was created in Step 3 by the instruction add r3, r1, r2.

Before Step 3:

r1 = 18
r2 = 6

Therefore:

r3 = r1 + r2
   = 18 + 6
   = 24

When Step 6 executes, the processor multiplies r3 by r2:

r6 = r3 × r2
   = 24 × 6
   = 144

Therefore, r3 holds 24 at Step 6 because the instruction add r3, r1, r2 in Step 3 calculated it by adding 18 and 6. This value is then used as a source operand by the multiplication instruction.

# Question 3 — Instruction Order

If the order of instructions 6 and 7 is swapped, the final value of r7 would not change.

Original order:

mul r6, r3, r2
add r7, r4, r5

Swapped order:

add r7, r4, r5
mul r6, r3, r2

Before these instructions execute, the relevant register values are:

r2 = 6
r3 = 24
r4 = 12
r5 = 12

The add instruction calculates:

r7 = r4 + r5
   = 12 + 12
   = 24

The mul instruction calculates:

r6 = r3 × r2
   = 24 × 6
   = 144

The multiplication only changes r6. It does not change r4, r5, or r7. Therefore, r7 remains 24 whether the multiplication is executed before or after the addition.

## Conclusion

The final value of r7 would not change because instructions 6 and 7 are independent of each other. Instruction 6 modifies r6, while instruction 7 uses r4 and r5 to calculate r7. Neither instruction modifies a register required by the other.

# Final Answers Summary

| Question | Answer | Key Value |
| --- | --- | --- |
| Question 1 | Final register state after 7 instructions | r1=18, r2=6, r3=24, r4=12, r5=12, r6=144, r7=24 |
| Question 2 | r3 was created by ADD in Step 3 and is used by MUL in Step 6. | r3 = 24 |
| Question 3 | Swapping instructions 6 and 7 does not affect r7. | r7 = 24 |